injury prevention

Quick Review of Safety

Ladder Safety

GENERAL LADDER SAFETY

- Always inspect the ladder before each use.

  - Look for broken rungs or rails; signs of corrosion; bad pulleys, ropes and locks.

  - Check the footings, pads and rungs to make sure they still provide a nonskid surface.

  - If any defect is found, the ladder should be tagged “unsafe” and taken out of service, fixed or disposed of properly.

- Always follow the manufacturer’s rated load capacity, including the weight of the user and equipment.

- Ladders should only be used for the purpose for which they were designed.

- Do not use ladders in doorways or other high-traffic areas without preparation. Consider potential hazards.

- Always set the ladder on a level and stable surface. Do not use other materials to raise the height of the ladder. If a ladder won’t reach, find a different ladder or other stable equipment.

- Always face the ladder when going up and down.

- Have at least two hands and one foot, or two feet and one hand in contact with the ladder at all times.

- Do not extend the midline of the body beyond the side rails of the ladder.

- Never use ladders horizontally like a platform.

- If tools are to be used, they should be carried in a tool belt or pulled up with a rope after you have reached your destination. Tools should not rest on steps of the ladder when not in use.

- Store ladders in a secure place where they cannot tip over.

- Do not step on the top cap or top step of a ladder.  Follow the manufacturer’s label and warning stickers about safe use.

(Over)

injury prevention

Quick Review of Safety

Ladder Safety

GENERAL LADDER SAFETY

- Always inspect the ladder before each use.

  - Look for broken rungs or rails; signs of corrosion; bad pulleys, ropes and locks.

  - Check the footings, pads and rungs to make sure they still provide a nonskid surface.

  - If any defect is found, the ladder should be tagged “unsafe” and taken out of service, fixed or disposed of properly.

- Always follow the manufacturer’s rated load capacity, including the weight of the user and equipment.

- Ladders should only be used for the purpose for which they were designed.

- Do not use ladders in doorways or other high-traffic areas without preparation. Consider potential hazards.

- Always set the ladder on a level and stable surface. Do not use other materials to raise the height of the ladder. If a ladder won’t reach, find a different ladder or other stable equipment.

- Always face the ladder when going up and down.

- Have at least two hands and one foot, or two feet and one hand in contact with the ladder at all times.

- Do not extend the midline of the body beyond the side rails of the ladder.

- Never use ladders horizontally like a platform.

- If tools are to be used, they should be carried in a tool belt or pulled up with a rope after you have reached your destination. Tools should not rest on steps of the ladder when not in use.

- Store ladders in a secure place where they cannot tip over.

- Do not step on the top cap or top step of a ladder.  Follow the manufacturer’s label and warning stickers about safe use.

(Over)

This document is intended for general purposes only and should not be construed as legal or coverage advice on any specific matter. Provided by Minnesota Counties Intergovernmental Trust.

This document is intended for general purposes only and should not be construed as legal or coverage advice on any specific matter. Provided by Minnesota Counties Intergovernmental Trust.

EXTENSION LADDER SAFETY

- Scan above and around you to assure that there are no hazards, such as electrical wires, that could touch you or the ladder.

- When using extension ladders, follow the one-to-four rule. This means if you are using a 12-foot ladder, the base should be three feet from the structure. Some ladders provide a picture on the ladder for reference.

- Extension ladders should be fully retracted before attempting to reposition them.

- Ladders should reach at least three feet above the point of support and should be secured when possible.

- Only make adjustments to the ladder when standing at the base, not when standing on the ladder or from a position above.

- Ensure locks are firmly engaged.

STEP LADDER SAFETY

- Before use, scan work area to ensure there are no hazards above or around you.

- When using a stepladder, make sure the folding cross braces are locked in the proper position before you step onto it.

- Do not use a stepladder like an extension ladder and lean against a wall. Stepladders should always be spread out fully.

EXTENSION LADDER SAFETY

- Scan above and around you to assure that there are no hazards, such as electrical wires, that could touch you or the ladder.

- When using extension ladders, follow the one-to-four rule. This means if you are using a 12-foot ladder, the base should be three feet from the structure. Some ladders provide a picture on the ladder for reference.

- Extension ladders should be fully retracted before attempting to reposition them.

- Ladders should reach at least three feet above the point of support and should be secured when possible.

- Only make adjustments to the ladder when standing at the base, not when standing on the ladder or from a position above.

- Ensure locks are firmly engaged.

STEP LADDER SAFETY

- Before use, scan work area to ensure there are no hazards above or around you.

- When using a stepladder, make sure the folding cross braces are locked in the proper position before you step onto it.

- Do not use a stepladder like an extension ladder and lean against a wall. Stepladders should always be spread out fully.