injury prevention

Quick Review of Safety

Aerial Lifts

BEST PRACTICES

- Conduct a thorough pre-start inspection based on the lift being used. Inspect the site for hazards, both on the ground and overhead.

- Always keep your feet on the floor of the platform or bucket; do not lean over or climb on guardrails.

- Never use ladders or other equipment to further extend your height from the lift platform.

- Always use fall restraint/arrest protection on the equipment that is attached to an identified location on the platform. A body harness should be worn and connected as soon as you or any other worker enters the platform or bucket.

- Always have a thorough understanding of the specific load capacity, wind tolerance, reach and other limitations of the lift to be used. Do not exceed these limits.

- Always be aware of what is in the travel path of the lift. Avoid electrical lines or other items within the path of travel; be aware of overhead objects as well.

- Never use aerial lift equipment in place of a crane to lift materials. Avoid oversized or awkward items on the platform or bucket; there should always be enough room for the worker to work with feet on the floor.

- Never travel with the lift raised more than what is given in the manufacturer’s instructions.

- Never travel with someone elevated in the platform or bucket unless the equipment is specifically designed for that use.

(Over)

injury prevention

Quick Review of Safety

Aerial Lifts

BEST PRACTICES

- Conduct a thorough pre-start inspection based on the lift being used. Inspect the site for hazards, both on the ground and overhead.

- Always keep your feet on the floor of the platform or bucket; do not lean over or climb on guardrails.

- Never use ladders or other equipment to further extend your height from the lift platform.

- Always use fall restraint/arrest protection on the equipment that is attached to an identified location on the platform. A body harness should be worn and connected as soon as you or any other worker enters the platform or bucket.

- Always have a thorough understanding of the specific load capacity, wind tolerance, reach and other limitations of the lift to be used. Do not exceed these limits.

- Always be aware of what is in the travel path of the lift. Avoid electrical lines or other items within the path of travel; be aware of overhead objects as well.

- Never use aerial lift equipment in place of a crane to lift materials. Avoid oversized or awkward items on the platform or bucket; there should always be enough room for the worker to work with feet on the floor.

- Never travel with the lift raised more than what is given in the manufacturer’s instructions.

- Never travel with someone elevated in the platform or bucket unless the equipment is specifically designed for that use.

(Over)

This document is intended for general purposes only and should not be construed as legal or coverage advice on any specific matter. Developed and provided by Minnesota Counties Intergovernmental Trust.

This document is intended for general purposes only and should not be construed as legal or coverage advice on any specific matter. Developed and provided by Minnesota Counties Intergovernmental Trust.

- If the lift has controls on both the platform and lower unit and a worker is on the platform, the lower controls should not be operated without the express consent of the worker on the platform except in an emergency.

- Always ensure that the lift is stable before beginning work. If equipment has outriggers, they should be positioned on a solid, level surface and the brakes should be set.

- If operations require the lift to be used on an incline, wheel chocks must be used if they can be safely installed.

- If the lift has controls on both the platform and lower unit and a worker is on the platform, the lower controls should not be operated without the express consent of the worker on the platform except in an emergency.

- Always ensure that the lift is stable before beginning work. If equipment has outriggers, they should be positioned on a solid, level surface and the brakes should be set.

- If operations require the lift to be used on an incline, wheel chocks must be used if they can be safely installed.